VIP-S Basic Instructions

Make sure the RESET/RUN switch is in the RESET position when powering on.

If you switch the VIP to RUN without holding down any Keypad button, the VIP-S will begin executing code at 0000 just like a standard VIP.  If you hold down a button when switching from RESET to RUN,  The VIP-S will begin to scan the keypad to determine which key has been pressed, then it will execute the program associated with that key.  Depending on which key you press, will determine your next steps.

There will be a few options:

Reset and hold C then release C and the VIP-S will run the standard RCA COSMAC VIP monitor.

Reset and hold D then release D  and the VIP-S will run Josh Bensadon's menu based serial monitor.  You will need a terminal connected to the TTL 5 volt serial connection to access the serial monitor.

Reset and hold E then release E and the VIP-S will copy CHIP8 from rom to  0x0000 and halt.  Toggle RESET/RUN again with C held and release.  Enter in the address 0200, then 0 to start entering your own CHIP8 program or use A to review your program.

Reset and hold F then release F to copy CHIP8 and Pin Ball to 0x0000 and 0x0200 respectively then halt. Then toggle the RESET/RUN switch again to run Pin Ball.

Reset and hold A then release A to copy CHIP8 and Animal Race to 0x0000 and 0x0200 respectively then halt. Then toggle the RESET/RUN switch again to run Animal Race.

Reset  with no keys pressed, and the JVIP will being executing what ever code is located at 0x0000

Josh’s Serial Monitor (RESET + D)

The following comments are slightly modified to reflect the usage of an FTDI instead of a standard serial port,  the rest is the same:

Communication for the serial monitor is done through the  FTDI port on the VIP-S.(Q for TXD, EF2 for RXD) a

successful connection is announced by sending it's Menu at 9600,N,8,1

VIP-S MENU:"COSMAC VIP-S says Hello (v1.a)"

" [D] Dump, [E] Enter, [R] Run ? "

Here, you may talk to your VIP-S via your terminal program.

D-Dumps Memory 1 page at a time, Enter an address, <Enter> dumps next page, <Esc> to quit.

E-Enters Data into the RAM, Enter an address, Enter Data, <Enter> to next byte, <Esc> to quit.

R-Runs at what ever address you select, Enter an address.

What's not shown on this menu is the ability to upload a .hex file!  Just go ahead and do a text

upload of your .hex file.  The ":" character will trigger the interpretation of the .hex line.

Here's a typical hex file line:

:1000000090B1B2B3B4F82DA3F83FA2F811A1D37206

iCCAddrRT----------DATA------------------CS

The : triggers the interpretation of the line.

The next two digits (10=16 bytes) give the count of bytes in that line

The next four digits (0000) sets the start address of the data bytes to come

The next two digits is the record type, 00 for data, 01 for end of file (no other types are interpreted)

The next 32 digits are the 16 data bytes in hex format

The last two digits are the checksum, which is tested for errors.  Any errors are counted.

After every line, the page counter is displayed on the TIL311 HEX Displays

Upon the 01 record type, the ERRORS counter is displayed and the monitor returns to the menu.

Now you can Run the program from the serial monitor with the R command, or you can reset the JVIP.

The standard VIP Monitor requires you to hold down the "C" key to enter the Monitor program.

Once the monitor program is entered, we tell the VIP-S we have 32K.

it then clears the last 80 bytes of ram in the last page of ram (0x0F00 in the JVIP) for a Video Display.

After you release the "C" button, you must enter a 4 digit starting address:  XXXX

Then select what to do as follows:

"0" to begin entering data into RAM, keep entering 2 digit hex values for consecutive RAM addresses

"A" to view memory, any key displays the next byte

"B" to read data from TAPE Drive (via EF2 line), enter the number of pages to READ * might switch this to read serial if possible?

"F" to write data to TAPE Drive (via Q line), enter the number of pages to WRITE * might switch this to write serial data if possible

RCA COSMAC VIP CDP18S711 Monitor ROM Listing

disassembled by Lee A. Hart <leeahart@earthlink.net>

Modified comments & added FTDI Routines for 9600,N,8,1

by Josh Bensadon

Reset then run the VIP-S to begin to use the device.

The ROM is partially decoded to occupy all memory

from 8000-FFFFh. Execution normally begins after

Reset with P=0, X=0, R(0)=0000, and the address

inhibit latch set so the monitor ROM is also at

0000-7FFFh instead of RAM. Thus the 1802 begins

executing the monitor ROM code.

register usage

R0 = 1861 video DMA pointer

R1 = 1861 interrupt handler PC

R2 = stack pointer

R3 = main PC

R4 = dismem subroutine PC

R5 = display subroutine PC

R6 = address pointer to read/write memory

R7 = getbyte subroutine PC

R8 = beeper timer (decremented by interrupt handler)

RA = bitmap table pointer

RC = key subroutine PC

.MSFIRST	 when using .dw, save Most significant byte first (Big Endian)

.org  8000h

LBR JVIPSTART

.org  8003h	 VIP MONITOR PROGRAM

here1:

sex r2

ldi 0		 new code added to clear R0

phi r0

plo r0

out 4            reset address inhibit latch (map RAM to 0000 area)

.db  0			   not required as it has already been done.

out 2            set keyboard latch to monitor "C" Key

.db  0Ch		   i dont think this is required because we already tested for c to get here.

ldi 0FFh         test for high end of RAM 32K

plo r1

ldi 7Fh

phi r1           set R1=7FFF (32k  max RAM on VIP)

notram:

ldi 0AAh         store AA

str r1

ldn r1           load it

xri 0AAh         if it worked,

bz  isram          then R1 points to top of RAM

ghi r1

smi 4            else R1=R1-0400 (try 1k lower)

bnf isram        if underflow, no RAM, continue anyway

phi r1

br  notram       if not 4k of RAM, then try 1k lower

isram:

b3  monitor

if "C" key is not pressed,

ghi r0             set R0=0000

plo r0

sex r0             set X=0, P=0,

sep r0             and go execute program at 0

R1=Last byte of RAM available (0FFF,0CFF,08FF or 04FF) (0FFF on VElf)

monitor:

sex r1

clear10:                 clear RAM from top=0xFF to 0xB0 of last page of ram (0FFF to 0FB0 on VElf)

ldi 0              clear a byte

stxd               decrement pointer

glo r1

xri 0AFh           until address=0xAF

bnz clear10

create RAM program to save registers

ldi 0D2h           push SEP R2 @rrAF

stxd

ldi 9Fh            push GHI RF @rrAE

str r1

glo r1           point R0 to RAM program

plo r0

ghi r1

phi r0

ldi 0CFh	 R1=rrCF

plo r1

M(rrCF) = RF.1

M(rrCE) = RE.1

M(rrCD) = RD.1

M(rrCC) = RC.1

M(rrCB) = RB.1

M(rrCA) = RA.1

M(rrC9) = R9.1

M(rrC8) = R8.1

M(rrC7) = R7.1

M(rrC6) = R6.1

M(rrC5) = R5.1

M(rrC4) = R4.1

M(rrC3) = R3.1

savereg:

sep r0           call the RAM program

stxd             push register contents it returns and dec X

dec r0

dec r0           point R0 to RAM program again

lda r0           decrement GHI Rx instruction

smi 1

dec r0           point R0 to RAM program again

str r0

xri 82h          if not down to GHI R2,

bnz savereg        then repeat

ghi r2           change PC to R3 (R2 was PC)

phi r3

ldi here2%256

plo r3

sep r3

here2:

ghi r0

phi r2		 R2.1 = rr (top page of ram) (0F on VElf)

phi rb		 RB.1 = rr (top page of ram) Page to load R0 with in video refresh

phi rd		 RD.1 = rr (top page of ram)

ldi 81h		 Set page of subroutines

phi r1		 R1=81xx

phi r4		 R4=81xx

phi r5		 R5=81xx

phi r7		 R7=81xx

phi ra           RA=81xx

phi rc		 RC=81xx

ldi int%256

plo r1           R1=8146 (1861 interrupt handler)

ldi 0AFh

plo r2           R2=rrAF

ldi dismem%256

plo r4           R4=81DD (display memory subroutine)

ldi display%256

plo r5           R5=81C6 (display hex char subroutine)

ldi getbyte%256

plo r7           R7=81BA (get hex byte subroutine)

ldi keyfound%256

plo rc           RC=81A1 (get key subroutine, Start at "KeyFound" since

the C key was pressed to get to this point)

getaddr:

sex r2

inp 1            turn on 1861 TV display

sep rc           wait until C key released

sep r7             get 1st key

sep r7             go back to get byte to shift and get 2nd key

sep r7             go back to get byte to OR the hex digits

phi r6           save in R6.1

sep r7             get 1st key

sep r7             go back to get byte to shift and get 2nd key

sep r7             go back to get byte to OR the hex digits

plo r6           save in R6.0

sep r4           display address R6, and M(R6)

sep rc           get a key

phi re             save it

bz  mwrite       if "0", go to Memory Write command

xri 10

bz  mread        if "A", go to Memory Read command

sep rc           get a key (count of pages to write/read to tape)

plo re             save it

dec r2

out 1            turn off TV

ghi re           get saved key

xri 0Bh

bz  tread        if "B", go to Tape Read command

ghi re           get saved key

xri 0Fh          if anything but "F",

bnz $              loop forever

twrite -- tape write command

ldi beep%256     change RC to point to Beep subroutine

plo rc

ldi 40h		 write lead in 0x40xx bits (~16,000 bits)

phi r9

twrite:			 write lead in loop

ghi r3		 r3 = PC = 0x8098

shr		 aka Clear DF flag

sep rc           call Beep to write 0 bit

dec r9

ghi r9

bnz twrite	 Loop ~16,000 times

twrite1:

ldi 10h		 Reset Parity Counter (Counts 1's sent)

plo r7

ldi 8

plo r9

lda r6		 Get byte to write to Tape

phi r7		 Hold Byte in R7.1

ghi r3

shl

sep rc           call Beep to write a 1 bit             -SEND START BIT (1)

glo r6		 Check for page boundary

bnz twrite2

dec re		 count down pages writen on every page boundary

twrite2:

ghi r7		 Fetch Byte to write

shr		 Shift out bit

phi r7

sep rc           call Beep to write a data bit          -SEND 8 DATA BITS

dec r9		 decrement bit counter

glo r9

bnz twrite2	 loop back to write 8 bits

inc r7		 Calculate EVEN Parity

glo r7

shr

sep rc           call Beep to write parity bit          -SEND PARITY BIT

glo re

bnz twrite1	 loop back to write next byte until all pages are written

sep rc           call Beep to write FINAL bit

tread6:

inp 1            turn on TV display

dec r6

sep r4           display last byte written

br  $            end  loop forever

tread -- tape read command

tread:

ldi tapein%256   change RC to point to tapein

plo rc             subroutine to read 1 bit

tread1:			 Lead In Loop

ldi 10

phi r9

tread2:

sep rc           call TapeIn to read 1 bit

bdf tread1	 Loop back while 1 (Lead IN)

dec r9

ghi r9

bnz tread2

tread5:

sep rc           call TapeIn to read 1 bit

bnf tread5	 Loop back until we get START BIT (1)

ldi 9

plo 9		 Set Bit Counter

plo 7		 Set Parity Counter

tread3:

ghi r7		 Get Byte to assemble

shrc		 Shift in READ bit

phi r7		 Save Byte

dec r9		 Count bits

sep rc           call TapeIn to read Data & Parity bits

glo r9

bnz tread3	 Loop back to read 9 bits (Shifts start bit out)

glo r7

shr

bdf tread4	 Jump if NO parity error

seq		 else, SET Q to indicate ERROR

tread4:

ghi r7		 Save BYTE

str r6

inc r6

glo r6

bnz tread5	 Test for Page boundary

dec re		 Count down pages to read

glo re

bnz tread5	 Jump back until all pages are read.

br  tread6       end  loop forever

mread -- memory read command

mread:

sep rc           get a key

inc r6           increment address in R6

sep r4           display address R6 and M(R6)

br  mread        repeat forever

mwrite -- memory write command

mwrite:

sep r7           get keys, assemble into byte

sep r7

sep r7

str r6

sep r4           display address R6 and M(R6)

(i.e. new contents of memory)

inc r6           increment memory pointer

br  mwrite       repeat forever

.db  0,0,0,0      unused?

index table -- converts hex digit into address in bitmap

table where that digit's bitmap begins.

.org 8100h

index:

.db  30h          "0" digit bitmap starting addr

.db  39h          "1"

.db  22h          "2"

.db  2Ah          "3"

.db  3Eh          "4"

.db  20h          "5"

.db  24h          "6"

.db  34h          "7"

.db  26h          "8"

.db  28h          "9"

.db  2Eh          "A"

.db  18h          "B"

.db  14h          "C"

.db  1Ch          "D"

.db  10h          "E"

.db  12h          "F"

bitmap table -- bit patterns to put on the screen to

display hex characters 0-F in a 5-high by 8-wide

format. Each pattern uses 5 consecutive bytes.

1's are bright, 0's are dark pixels. The patterns

overlap to save memory.

bitmap:

.db  11110000b    top of "E"

.db  10000000b

.db  11110000b    top of "F"

.db  10000000b

.db  11110000b    top of "C"

.db  10000000b

.db  10000000b

.db  10000000b

.db  11110000b    top of "B"

.db  01010000b

.db  01110000b

.db  01010000b

.db  11110000b    top of "D"

.db  01010000b

.db  01010000b

.db  01010000b

.db  11110000b    top of "5"

.db  10000000b

.db  11110000b    top of "2"

.db  00010000b

.db  11110000b    top of "6"

.db  10000000b

.db  11110000b    top of "8"

.db  10010000b

.db  11110000b    top of "9"

.db  10010000b

.db  11110000b    top of "3"

.db  00010000b

.db  11110000b

.db  00010000b

.db  11110000b    top of "A"

.db  10010000b

.db  11110000b    top of "0"

.db  10010000b

.db  10010000b

.db  10010000b

.db  11110000b    top of "7"

.db  00010000b

.db  00010000b

.db  00010000b

.db  00010000b

.db  01100000b    top of "1"

.db  00100000b

.db  00100000b

.db  00100000b

.db  01110000b

.db  10100000b    top of "4"

.db  10100000b

.db  11110000b

.db  00100000b

.db  00100000b

interrupt routine for 64x32 format (1 page display memory)

intret0:

req              return with Q off

intret1:

lda r2

ret              <-return with interrupts enabled

int:                     ->entry with P=R1

dec r2           point to free location on stack

sav                push T

dec r2

str r2             save D

nop              3 cycles of NOP for sync

inc r9

ldi 0            reset DMA pointer to start of

plo r0             display RAM

ghi rb

phi r0

sex r2           NOPs for timing

sex r2           set D=line start address (6 cycles)

disp:

glo r0

1861 displays a line (8 cycles)

sex r2

sex r2           reset line start address (6 cycles)

dec r0

1861 displays line a 2nd time (8 cycles)

plo r0

sex r2           reset line start address (6 cycles)

dec r0

1861 displays line a 3rd time (8 cycles)

plo r0

sex r2           reset line start address (6 cycles)

dec r0

1861 displays line a 4th time (8 cycles)

plo r0           set R0.0=line start address

bn1 disp         loop 32 times

ghi r8		 r8.1 is TIMER,  Counts down when set>0

bz  disp1        if Zero, skip decrementing

plo rb		 (Using rb.0, to avoid altering DF)

dec rb             else decrement beeper time remaining

glo rb               ...high byte

phi r8

disp1:

glo r8               ...low byte

bz  intret0      if beep time is zero, then return with Q off

seq                else return with Q on

dec r8

br  intret1

beep -- output a 1-cycle square wave on Q

at 2 KHz if DF=0, or 0.8 KHz if DF=1.

Also increments R7.

beepx:

sep r3           <-return

beep:

ldi 10           2 KHz if DF=0

bnf setq

ldi 32           0.8 KHz if DF=1

inc r7		 Parity Bit count the 1's

setq:

seq              set Q    16 cycles

phi rf           save delay length 16 cycles

beepl:

smi 1            delay 16 clock cycles  @3.579/2 = 1.79Mhz = 0.558uSec

bnz beepl	       16 clock cycles, =(32*10+48)*0.558uSec=205uSec=(1/2 cycle of 2.44Khz)

=(32*32+48)*0.558uSec=598uSec=(1/2 cycle of 836hz)

@3.52/2 = 1.76Mhz = 0.568uSec

16 clock cycles, =(32*10+48)*0.568uSec=209uSec=(1/2 cycle of 2.39Khz)

=(32*32+48)*0.568uSec=609uSec=(1/2 cycle of 821hz)

bnq beepx        exit after low half cycle 16 cycles

req              reset Q

ghi rf		 fetch delay length

br  beepl        delay again

tapein -- cassette tape audio input  read one cycle.

If low frequency, returns DF=1 and increments R7.

If high frequency, returns DF=0.

tapeinx:

sep r3           <-return

tapein:

ldi 16           set timer

bn2 $            wait for EF2=1 (EF2 pin low)

(i.e. wait for rising edge)

tapein1:

bn2 tapein2      wait 4 x 16 cycles and test EF2

smi 1

bnz tapein1      if it times out, is low frequency

inc r7             increment R7 Parity Counter

ghi rc             and set D=81h (to set DF)

tapein2:

shl              shift high bit of D into DF

if DF=1, is low freq

if DF=0, is high freq

b2  $            wait for EF2=0 (EF2 pin high)

(i.e. low half of this cycle)

br  tapeinx        and exit

get a key -- scans keypad, and returns when a key is found.

'Beeps' while key is down. Returns key (0-F) in D and

on the stack. Uses RC as its dedicated program counter.

key:

sep r3           <-return

GETKEY:

sex r2           ->entry: scan keypad, return when found

ghi rc

plo rf           for n = 10h to 0:

keyn:

dec rf             decrement n

dec r2

glo rf             set keypad latch to n

str r2

out 2              is key = n?

sex r2

sex r2

bn3 keyn           until key found

keyfound:

ldi 4            ->enter with P=RC

plo r8           set beep duration to 4/60 sec

glo r8

bnz $-1          wait for it to end

ldi 4

plo r8           keep beeping until key released

b3  $-3            (i.e. EF3 pin returns high)

glo r8

bq  $-3

glo rf

ani 0Fh          mask key to 0-F

str r2             and return it on the stack

br  key

.db  0,0,0,0      (unused)

getbyte -- get 2 keys from the keypad, and assemble them

into a byte. Return the byte in D. Uses R7 as its

dedicated program counter.

sep r3           <-return

getbyte:

sep rc           ->enter  get 1st key

shl

shl              shift it into high nibble

shl

shl

plo re           save it

sep rc           get 2nd key

glo re

or               combine them and return in D

br  getbyte-1

display -- display hex digit & advance pointer. On entry:

D = display table base addr + hex digit.

RD pointer to video RAM addr of top line of char.

Uses R5 as its dedicated program counter.

sep r4           <-return

display:                 ->entry

plo ra           point RA to display table + digit

ldn ra           get starting addr of char gen.

plo ra           point RA to top line bit pattern

ldi 5            characters are 5 lines high

plo rf             RF.0=line counter

display1:

lda ra             get display pattern

str rd             put in video RAM

glo rd             add 8 to video RAM address

adi 8                to point to next line of char

plo rd

dec rf             decrement line counter

glo rf             get line counter

bnz display1       loop unti last line done

glo rd

adi 0D9h         set RD back to original video RAM

plo rd             address +1 (points to next char)

br  display-1    and return

dismem -- display memory address and data in hex. On entry,

R2 (stack pointer) points to an occupied byte

R6 = address, and is pointing to data byte to display

Uses R4 as its dedicated program counter.

dismemx:

sep r3           <-return

dismem:

dec r2           ->entry

ldn r6           push data byte to display onto stack

stxd

glo r6           push low byte of address onto stack

stxd

ghi r6           store high byte of address on stack

str r2

ldi 6            display 6 hex digits

plo re

ldi 0D8h         RD=position in video RAM

plo rd

dismem1:

ldn r2           get hex byte from stack

shr

shr              shift out low nibble

shr

shr

sep r5           display high nibble

lda r2           pop hex byte from stack

ani 0Fh          mask out high nibble

sep r5           display low nibble

glo re

shr

plo re

bz  dismemx      if RE.0=0, exit

bnf dismem1      if DF=0, repeat for another byte

(i.e. is address, which takes 2 bytes)

inc rd           else move right 2 positions in video RAM

inc rd             to put space between address and data

br  dismem1        and go display 2 more digits (data byte)

.db  1            checksum?

.org  8200h

JVIPSTART:

sex r0

out 2           	 set keyboard latch to monitor keypress

.db  0Ch			 check for "C" being pressed

B3 VIPMONLBR		 branch to code that long brances to VIP Monitor ram test

out 2				 set keyboard latch to monitor keypress

.db 0Dh				 check for "D" being pressed

B3 JBSERIALMONLBR	 branch to code that long brances to Josh's Serial Monitor Code.

out 2          		 set keyboard latch to monitor keypress

.db  0Eh			 check for "E" being pressed

B3 LDCHIP8LBR		 branch to code that long brances to the code that loads CHIP8 code to 0x0000

out 2				 set keyboard latch to monitor keypress

.db 0Fh				 check for "E" being pressed

B3 LDPINBALLLBR	 branch to code that long brances to the code that loads CHIP8 code to 0x000  and loads Pinball to 0x0200

br JVIPSTART			 Keep scanning until of of the above keys are pressed.

We don’t loop, we just branch to 0000 to start running code right?

VIPMONLBR:

ldi here1/256   set R2=8003 as per Original VIP ROM

phi r2

ldi here1%256

plo r2

sep r2 			 long branch to VIP monitor (past the code that checks for C being pressed, start at ram test and address entry)

JBSERIALMONLBR:

LBR 8300h			 Long Branch to Josh's serial monitor that was relocated to 0x8300

LDCHIP8LBR:

LBR 8400h			 long branch to the code that transfers CHIP8 into ram at 0x0000 then runs VIP monitor code waiting for data entry

LDPINBALLLBR

LBR 8600h			 long branch to the code that transfers CHIP8 and Pinball into ram at 0x0000 and 0x02000 respectively

HOMERUN:

LBR 0000h			 long branch to 0x0000

.end